| #### SEALANT TAPE | #### LTS90B & G |
| --- | --- |

### DESCRIPTION

Economic low cost vacuum bag sealant tape, excellent tack.

Ideally suited to debulking operations at ambient temperatures and high temperature envelope bagging applications.

Material compatible with most substrates (including composite tools) and film materials.

Tape does not strip cleanly from any tool surface when warm.

### PHYSICAL PROPERTIES

Use Temperature		Up to 150C bag to tool* (see text)

Up to 180C bag to bag

Color				Black (LTS90B), GREY (LTS90G)

Solids			100%

Flammability			Not flammable but combustible

(Note: If tape is used on bag to tool surface applications the tape must be left to cool to ambient before attempting to strip it off)

### AVAILABILITY & PACKAGING

Standard rolls, 15 meters long x 12mm x 3mm are packed in cardboard cartons containing 22 rolls.

Also available as LTS90ROUND.

### STORAGE & HANDLING

LTS90 is a low toxicity, non-irritant and non-injurious to skin.  It should be stored in its original containers under ideal conditions, between 5 and 25C. The material has a 1-year shelf life.  Material must be stored flat.

All statements, technical information and recommendations contained in this publication are based on tests believed to be reliable, but their accuracy and/or completeness are not guaranteed. The user shall determine the suitability of the product for this particular purpose and shall assume all risk and liability in connection herewith.

ISSUE DATE: OCTOBER 2000

P.O.Box 93            Tel.  +358 9 681 081

FIN 00211           Fax. +358 9 692 4174
HELSINKI